Problem 1:

The pump shown in Figure 1 is used to pump water from one open reservoir up to another open reservoir for storage. The top surface of the upper reservoir is located 55 m above the top surface of the bottom reservoir and is fed by a 60 cm diameter pipe. The ﬂow rate through the pipe is held at a constant 3.0 m3/s. The frictional losses through the pipe can be computed using the equation

Where f is a dimensionless loss factor (obtain with Haaland equation or Moody chart, using

∈D = 0.001

Assuming negligibly small minor losses (entry and exit and elbows) and the pump operates with an eﬃciency of 80 %, what is the required input power to run the pump?

Figure 1: Reservoir and pump system sketch.

#### Water density at 2020 degree centigrade (shown in figure 1) = ρ = 998.2 𝑘𝑔𝑚/3

#### Water dynamic viscosity at 2020 degree centigrade (shown in figure 1) = µ = 0.0010020 𝑘𝑔𝑚/𝑠.

#### Pipe Internal Diameter = d = 0.8 m

#### Fluid velocity = v

Note: Equations are snapshots taken from Jupyter Notebook:

Problem 2:

Vapor in a multi-effect distillation plant such as the one shown in Figure 2 vapor can carry mist droplets with salt and other contaminants. To limit the amount of contaminants that reach the vapor exit (see arrow in the figure), we may want to control the speed of the vapor so that it can carry only the liquid droplets smaller than a certain size upwards.

Question: If we have a vertical ow of saturated water vapor at 0.15 bar absolute pressure and we want to allow it to carry droplets with a maximum diameter of 120μm, what is the maximum velocity at which the vapor can flow to not carry larger droplets?)

Assumptions and hints:

- Assume steam is saturated. Definition: Water heated to boiling point is vaporized and turns to saturated steam. If saturated steam is heated further (temperature above boiling point at given pressure), dry steam is generated and traces of moisture are gone (and in such case we have superheated steam).

- Assume droplet is spherical and can be treated as a rigid sphere a first approximation.

- Use the density and viscosity of liquid and vapor water at the appropriate saturation temperature. One reference for such properties can be found here: NIST - Thermophysical Properties of Fluid Systems.

- Hint: start with a force balance. Assume that droplet would need to travel vertically upward (towards the screen above).

## Solution 2

Coefficient of drag for rigid sphere:-

The Maximum velocity at which the vapor can flow to not carry larger droplets (𝑈_𝑣 =? )

## Force Balance Formula

Problem 3:

To have a robust yet economical mass ﬂow rate measurement with a relatively low pressure loss, a venturi ﬂow meter may be utilized. Let us consider the performance and likelihood of cavitation in a horizontal venturi. We shall assume constant ﬂuid properties, and denote the contraction ratio as β, deﬁned as β = dthroat/dpipe. For a venturi, the mass ﬂowrate can be determined as a function of ﬂuid density, measured diﬀerential pressure ∆p = Ppipe – Pthroat and contraction ratio β.

(A) Sketch the venturi and label locations where you deﬁne pressures.

- velocities in the Equations:

m=Athroat2ρ∆ρ1-β4

β=d2d1<1

∆ρ=ρpipe-ρthroat>0

B) As no instrument is perfect; from the lectures we recall that above equation needs to be multiplied by a discharge coeﬃcient. Assume it has a constant value of Cd = 0.965. Our venturi has β = 0.6 and Athroat = 0.003 m2. We shall assume that the uncertainty in density is 2 %, in area of throat 1 % and in β is 1 % for each, all with 95 % conﬁdence. If mass ﬂow rate of 25 kg/s is to be determined within 5 % with 95 % conﬁdence, what is maximum uncertainty allowable for the diﬀerential pressure measurement?

Results

∆Pmean= 32509.85 Pa

∆Pstd. deviation=3384.06Pa

Maximum uncertainty allow is 3384.06Pa32509.85 PaX100=10.40%

Results

∆ρmean=32531.12 Pa

∆ρstd. deviation=33241.77  Pa

Maximum uncertainty allow is 33241.77Pa32531.12 PaX100=10.22%

(C) If cavitation inception is expected at σ=ρref-ρvapor12ρUref2≤1.8, with cavitation number deﬁned based on ’reference’ pressure and velocity upstream of the throat. For the above ﬂow rate, what is minimum pressure allowed at the venturi inlet to likely avoid cavitation? Assume water temperature is 20 ◦C.

Hint: start by deﬁning the cavitation number and be consistent with labeling from part A.

- σ=P-Pvapor12ρUref2≤1.8

- σ Ppipe-Pvapor12ρvpipe2=1.8

Solve for vthroat:

- vthroat=mρAthroat

- vthroat=25998.290.003

- vthroat=8.34 m/s

Solve for vpipe:

- vpipe=vthroat β2

- vpipe=8.340.62

- vpipe=3.0m/s

Minimum Allowed Pressure at Venturi Inlet to avoid Cavitation

∴ Pvapor=2353

∴given that σ=1.8.

Know solve Ppipe:

- Ppipe=σ12ρvpipe2+Pvapor

- Ppipe=1.8.12 . ρ . vpipe2+Pvapor

- Ppipe=1.8.12 . (998.29) . 3.02+2353

- Ppipe=8086.149+2353

- Ppipe=10439.149Pa ≈10.43KPa

Problem 4

Assume we have a 600 mm ID pipeline that is 300 m long supplying water to an older desalination plant at a mass ﬂow rate of 850 kg/s. Due to corrosion the average roughness of the pipe has increased to 5.5 mm, from the original 0.1 mm. Assume pumping eﬃciency of 82 %, electricity price of 0.11 USD/kWh. And, that you are pumping seawater at 20 ◦C.

Also note: the liner could also block leaks the pipeline may have had.

Problem 5

All other things being equal, smoother pipe, larger diameter, more gradual pipe transitions etc. will lead to a reduced pressure drop in pipe network. And, ideally none of the components should be inﬂicted by cavitation. During the course we discussed pressure loss estimation based of textbook approaches, and calculations nowadays typically handled by commercial codes. For this assignment, you will examine a simple pipe network and study eﬀect of uncertainty to network performance using the Monte-Carlo method for uncertainty propagation.

Use a textbook approach (with friction factors e.g. from Moody diagram (Figure 6.13 in White) based on given roughness White (1998)) to simulate the performance the network should have. Note that simulated performance may deviate from measured performance not only due to uncertainty about initial dimensions of components, but also due to installation deviating from design (e.g. extra bends and rough welds), eﬀects of corrosion and fouling on roughness given age of pipeline, etc.

For a complex pipe network, commercial or freeware codes would typically be used to evaluate performance of the network. While the solution methodologies vary, the basic conservation laws remain unchanged. Hence, let us consider a simple example pipe network and a) write our own code to solve for the ﬂow rates, b) use WNTR simulation library in python to resolve the same network and ensure your answers match, and c) implement a Monte-Carlo solution.

Assume pipe diameters have uniform distribution between 98% and 103% of nominal diameter and roughness has an exponential probability distribution with mean being the listed value. Rerun the Monte Carlo simulation with water properties at 10◦C and 30◦C (in addition to the original design point of 20◦C) in order to evaluate the inﬂuence of temperature on system performance. NOTE: The WNTR simulation library in python will be used for comparison in

Figure 3: Pipe network to analyze. Figure adapted and modifed from Jeppson (1974), and the

values have been changed.

a) Use whichever method you are most comfortable with (Linearized approach from lectures, Hardy-Cross, Newton-Rhapson, or other) to setup equations and then solve the ﬂow rates and pressures throughout the network. Start from conservation of mass and consider only the major losses for ∆p calculations(i.e. neglect minor losses due to ﬁttings and instruments).

Very Small Error Values

c)Use WNTR to check solution of part a. Report your results for ﬂow rates and pressures throughout the network.

Hint: WNTR requires at least one reservoir, so even though we know all of the ﬂowrates into the network, you will need to set one as a reservoir. WNTR will solve for the ﬂow from the reservoir into the network and you can check to conﬁrm that it matches the given value and mass is con-served. Also, make sure that all of your junctions and pipes have unique names.

Check point: To check your result, the ﬂowrate in pipe 2 should be approximately 0.160 m3/s and the head at node 3 should be approximately 6.3 m.

Pressure Conv and Power Conv

g) Within each pipe, calculate viscous length scale (SSA will provide example upon request), wall shear stress and estimate if a super hydrophobic coating with damage threshold of 10 Pa of shear and manufactural with rams roughness of 100 µm might result in drag reduction in each pipe segment (Requires having roughness below ∼ 5 viscous length scales. For simplicity: assume that there would be a way to avoid entrainment of gas from the surface into the ﬂow and a way to supply gas to the surface at pipeline pressure – not necessarily achievable presently).